# Decision-Centred Modelling

2026

Abstract

Long-horizon infrastructure decisions face a structural analytical problem distinct from insufficient physical detail: when a model draws its boundary around the physical system being analysed, the consequences that cross that boundary — infrastructure-conditional costs, cross-scale demand propagation, regional network constraints — become analytically inaccessible. For decisions where these consequences are material, the model may point in the wrong direction.

This paper introduces the Decision-Centred Modelling (DCM) framework, which addresses this problem by drawing the analytical boundary around the decision and its consequences rather than around the physical system. The framework rests on three principles: a decision-first boundary specification that defines analytical scope through decision-relevance; a thin-waist artefact governance layer coupling heterogeneous analytical modules through schema-conforming exchange contracts; and progressive refinement governed by regret diagnostics. Under conditions where probability distributions over futures cannot be defended, regret, robustness, and satisficing replace expected value optimisation as the appropriate evaluative standards.

Applied to industrial process heat decarbonisation in Southland, New Zealand, the framework reveals a material difference between site-boundary and decision-boundary analysis. From within the facility boundary, a biomass switching pathway (BB) has a NZD 14.2M/year private cost advantage over full electrification (EB), with no visible infrastructure consequence. When regional grid infrastructure costs are included within the decision boundary, BB is preferred in 93 of 100 plausible futures — while EB exposes the decision to a major grid reinforcement class in 49 of those futures, a consequence invisible to conventional site-boundary analysis. The framework architecture is domain-general. The platform and proof-of-concept data are openly available at https://ahmad-mahmoudi-coder.github.io/DCM/.

## 1. Introduction

Long-horizon infrastructure decisions share a structural feature that is underappreciated in the modelling literature: the consequences that determine whether a decision was well-made often extend beyond the physical boundary of any single system. A factory that commits to electrifying its process heat creates load that propagates through the regional electricity network. A region that pursues industrial electrification generates infrastructure demand that affects grid upgrade timelines and tariff structures for all connected users. These consequences are not incidental — they are frequently decisive. Yet they fall outside the scope of the analytical models most commonly used to support these decisions.

The problem is not computational capacity. Modern energy system models — TIMES, PyPSA, MESSAGE, Calliope — can represent extraordinary technical complexity. The issue is boundary placement. When an analytical model draws its boundary around the physical system being optimised, it cannot represent consequences that cross that boundary, however material those consequences may be. This is not a failure of the models — they were not designed to represent these consequences — but it creates a systematic gap between what the model can see and what the decision actually requires.

This paper introduces the Decision-Centred Modelling (DCM) framework, which addresses this gap by repositioning the analytical boundary: rather than drawing it around the physical system, the framework draws it around the decision and its decision-relevant consequences. Where a system-centred model asks “what is the least-cost technology for this facility?”, a decision-centred model asks “how does this technology choice perform, across the full range of consequences and futures, relative to available alternatives?” These are different questions with structurally different analytical requirements.

The framework makes three claims. First, an architectural claim: a governed, modular analytical environment organised around the decision problem and coupled through thin-waist artefact exchange is technically feasible and analytically better suited to multi-scale infrastructure decisions under deep uncertainty than monolithic system-centred models. Second, a methodological claim: under conditions where probability distributions over futures cannot be defended, regret, robustness, and satisficing are more appropriate evaluative standards than expected value optimisation. Third, an empirical claim: when the Edendale industrial electrification decision is evaluated within the DCM framework — including regional grid infrastructure consequences within the decision boundary — the comparison between pathways changes materially relative to a conventional site-only analysis, in 93 of 100 plausible futures.

Section 2 reviews the adjacent traditions against which the framework is positioned. Section 3 specifies the framework design. Section 4 describes the uncertainty architecture. Section 5 presents the Southland proof of concept. Section 6 discusses implications, limitations, and generalisability. Section 7 concludes.

## 2. Background and Related Work

### 2.1 Energy System Modelling

The energy system modelling literature offers a mature set of tools for evaluating technology transitions and policy pathways. TIMES and its national implementations use linear programming to identify least-cost national energy system configurations over multi-decade horizons (Herbst et al. 2012). PyPSA provides open-source network optimisation for electricity systems (Pfenninger et al. 2018). Calliope and similar capacity expansion models enable multi-sector analysis with explicit spatial and temporal resolution.

These tools are valuable for the purposes they were designed to serve. Two characteristics become limiting specifically for site-to-regional industrial decarbonisation decisions. First, they draw their analytical boundary around the physical system being optimised. Consequences that cross that boundary — GXP-level hosting capacity constraints, infrastructure- conditional cost propagation from site to network — are not represented because they are outside the model’s declared scope. Second, they handle uncertainty through scenario comparison. TIMES-NZ 3.0, New Zealand’s primary national energy model, evaluates two structural scenarios (Steady and Shift) that span four critical uncertainties (Energy Efficiency and Conservation Authority 2024b). Two scenarios provide valuable macroeconomic framing but cannot span the multi- dimensional uncertainty space that a twenty-year industrial infrastructure commitment faces across multiple independently varying drivers.

These are not critiques of the models for their stated purposes. They are observations about the specific analytical gap that arises when site-to-regional multi-scale decisions require more than national-scale scenario analysis can provide.

### 2.2 Decision Making Under Deep Uncertainty

DMDU methods emerged because conventional scenario analysis was producing inadequate guidance for long-horizon planning problems (Marchau et al. 2019). Knight’s distinction between risk — where probabilities are knowable — and uncertainty — where they are not (Knight 1921) — identifies the structural condition. Under deep uncertainty, Simon’s satisficing (Simon 1955) is the appropriate standard: find alternatives that perform acceptably across many futures rather than optimising under one assumed future.

Robust Decision Making (Lempert et al. 2003) operationalises this through ensemble evaluation: alternatives are assessed across large sets of plausible futures using regret and robustness rather than expected value. Dynamic Adaptive Policy Pathways (Haasnoot et al. 2013) designs decision sequences that can be revised as conditions evolve. Exploratory Modelling and Analysis (Kwakkel et al. 2016) provides the computational infrastructure for large-scale ensemble evaluation.

The DMDU tradition provides the evaluative standards the DCM framework adopts. Its primary limitation for the present purpose is the absence of a modular architectural specification that makes these standards tractable at multi-scale resolution for industrial infrastructure decisions. The DCM framework addresses this gap.

### 2.3 Sequential Decision Analytics

Powell’s unified framework for sequential decision analytics (Powell 2022) classifies any decision method into four policy classes: policy function approximations (PFAs), cost function approximations (CFAs), value function approximations (VFAs), and direct lookahead approximations (DLAs). His formulation searches over both the function class f∈F and parameters θ∈Θ to maximise expected cumulative contribution. The DCM framework uses a CFA — proportional dispatch — for computationally tractable ensemble evaluation across 100 futures.

The critical extension the DCM framework makes is applicable when the expectation operator in Powell’s formulation cannot be applied: when probability distributions over futures are not available or defensible. In that condition, regret and satisficing replace E[⋅] as the evaluative standard. Powell’s policy class taxonomy remains applicable; the evaluation metric changes.

### 2.4 The Analytical Gap

The gap the DCM framework addresses arises at the intersection of three conditions that existing approaches do not jointly satisfy: multi-scale consequence visibility spanning site, regional, and national levels; ensemble-based deep uncertainty evaluation; and modular, reproducible, auditable analytical architecture. A systematic review of modelling tools for industrial process heat decarbonisation found no existing tool met all three conditions jointly (Lahijani et al. 2025). The DCM framework was designed to fill this gap.

## 3. Framework Design

### 3.1 The Decision-First Boundary Principle

The foundational architectural commitment of the DCM framework is that analytical boundaries are drawn by the decision context — not by the physical extent of the system being modelled. The decision-relevant boundary is defined as the smallest analytical scope within which all consequences material to the comparison of alternatives are visible.

This principle has a specific operational implication: before any analytical module is designed, the decision context must be declared. What decision is being supported? What pathways are being compared? What consequences are material to that comparison? What scales do those consequences span? The answers define the analytical scope. The physical system boundary is then one layer within that scope, not the boundary of the analysis.

For the industrial electrification decision in §5, the material consequences include site operating costs, regional grid infrastructure upgrade requirements, and carbon cost exposure — spanning facility, GXP, and national policy scales simultaneously. A model drawn at the facility boundary can represent the first; it cannot represent the second or third without deliberate architectural extension.

### 3.2 Modular Architecture and Thin-Waist Coupling

The DCM framework is organised as a seven-layer analytical stack, from facility demand construction through site dispatch, GXP interface, regional electricity modelling, policy context, DMDU ensemble evaluation, to the decision layer. Each layer is a distinct analytical module that can be developed, validated, and replaced independently.

The coupling mechanism between layers is a thin-waist artefact exchange. The thin-waist principle — from internet protocol design, specifically the end-to-end argument in network architecture (saltzer1984end?) — specifies that inter-module interfaces must be narrow (carrying only what must cross the boundary), explicit (precisely schema-specified), and stable (unchanging even as module internals evolve). In the DCM framework, every output that crosses a module boundary is a schema-conforming, provenance-carrying, validation-gated artefact carrying its schema version, the inputs that produced it, a SHA-256 hash for integrity verification, and the validation record that admitted it into the comparison chain.

This architecture produces two analytical properties. First, genuine reproducibility: every finding is traceable to every assumption through the artefact chain, and any module can be replaced without disturbing the chain provided the interface contract is preserved. Second, tractable progressive refinement: modules can be developed to minimum viable specification and improved where regret diagnostics reveal that improvement would change the decision-relevant comparison.

### 3.3 Evaluation Standards Under Deep Uncertainty

The framework evaluates pathway alternatives using three complementary metrics that jointly characterise decision quality when probability distributions over futures cannot be defended.

Regret of pathway s under future ω:

ρ(s,ω)=Z(s,ω)−mins′∈SZ(s′,ω)

where Z(s,ω) is the total system cost including site operating costs and annualised infrastructure costs. Regret is zero for the minimum-cost pathway in each future. Maximum regret Rmax(s)=maxωρ(s,ω) characterises the worst-case cost of choosing s.

Satisficing rate at threshold τ — the fraction of futures in which total cost falls within an acceptable range:

SR(s,τ)=|{ω:Z(s,ω)≤τ}||Ω|

Win rate — the fraction of futures in which pathway s achieves the minimum system cost:

WR(s)=|{ω:ρ(s,ω)=0}||Ω|

Win rate is technically implied by regret (zero regret equals a win) but provides a distinct frequency characterisation useful for communication. A pathway with high satisficing rate, low mean regret, and low maximum regret is preferred under deep uncertainty — regardless of which futures are most likely, and without requiring probability distributions over Ω.

### 3.4 Progressive Refinement

The framework’s development philosophy is governed by regret diagnostics rather than completeness aspirations. A minimum viable implementation — honest about its limitations, architecturally complete, and analytically rigorous within its declared scope — is more valuable than a comprehensive but opaque system.

Regret diagnostics operate as follows. For each uncertain driver in the ensemble, the sensitivity of the pathway comparison to that driver’s value is computed. Drivers that produce the largest swing in pathway preference — measured by the range of regret values across futures sorted by that driver — are the highest-priority development targets. In the Edendale PoC, the GXP headroom multiplier produces the largest regret sensitivity: futures with lower headroom are those where EB most frequently triggers the N+150 MW upgrade class. This identifies regional network modelling fidelity as the highest-priority analytical investment — not for methodological reasons, but because it is the uncertain driver most likely to change the recommendation if represented with greater fidelity. The framework is in this sense self-directing: it reveals its own most productive next development step.

## 4. Uncertainty Architecture

Long-horizon decisions face three qualitatively distinct types of uncertainty requiring different analytical treatments. This three-layer taxonomy extends Knight’s risk/uncertainty distinction (Knight 1921) by specifying operationally distinct regimes and connecting each to a tractable analytical response.

Layer 1 — Parametric uncertainty covers conditions where uncertain drivers can be characterised by plausible ranges and sampled via structured ensemble methods. Capital cost multipliers, electricity tariff trajectories, carbon price scenarios, and demand growth rates are parametric uncertainties. Their ranges can be calibrated from historical data and the TIMES-NZ 3.0 Steady/Shift scenario bounds. The PoC demonstrates Layer 1 treatment through Latin Hypercube Sampling across five drivers.

Layer 2 — Structural uncertainty covers conditions where the world-model itself is contested — not parameter values but the structural assumptions of the analytical framework. Whether LNG import infrastructure is developed or gas exits the energy mix; whether dairy export demand follows the Steady or Shift trajectory. These are discrete branches in the space of possible futures, not points in a continuous parameter space. The TIMES-NZ 3.0 Steady/Shift scenarios represent structural uncertainty at the national level, providing calibrated bounds (52/tCO2 to 260/tCO2) for the ETS price dimension (Energy Efficiency and Conservation Authority 2024b).

Layer 3 — Deep narrative uncertainty covers conditions that are difficult to parameterise with defensible probability assignments: geopolitical ruptures, pandemic-scale disruptions, regulatory reversals. These can only be evaluated as stress tests — whether the preferred decision retains its advantage if the shock occurs. The robustness-first evaluation philosophy addresses Layer 3 indirectly: decisions that perform well across Layers 1 and 2 tend to be more resilient to Layer 3 shocks than decisions optimised for a single assumed future.

The current PoC demonstrates Layer 1 treatment only. Layer 2 structural scenario branching and Layer 3 stress testing are specified in the framework architecture and represent natural extensions of the current implementation.

## 5. Proof of Concept: Edendale–Southland

### 5.1 Decision Context

The Fonterra Edendale dairy processing facility in Southland, New Zealand, is one of New Zealand’s largest industrial process heat consumers, with an annual heat demand of approximately 819 GWh (Energy Efficiency and Conservation Authority 2024a). Coal provides approximately 80% of process heat at baseline (2020). New Zealand’s National Environmental Standards mandate coal phase-out from industrial process heat by 2037. Two primary decarbonisation pathways are available: full electrification via electric boilers (EB pathway) or biomass substitution via biomass boilers (BB pathway). The decision involves a capital commitment with a 20+ year operational life and consequences that propagate to the regional electricity network.

### 5.2 Analytical Pipeline

The PoC implements five of the seven DCM layers.

Demand construction (Layer 1): A synthetic DemandPack is constructed for each planning epoch (2020, 2025, 2028, 2035) from EECA’s Regional Heat Demand Database (Energy Efficiency and Conservation Authority 2024a), representing hourly heat demand at 8,760-timestep annual resolution with seasonal and operational variability.

Site dispatch (Layer 2): Proportional dispatch allocates heat demand across committed technology units in proportion to nameplate capacity — a cost function approximation (CFA) in Powell’s policy class taxonomy (Powell 2022). Annual costs include fuel, electricity tariffs, NZ ETS carbon costs, and fixed and variable operation costs.

GXP interface (Layer 3): The SignalsPack carries the hourly incremental electricity demand: the difference between EB or BB electricity import and the 2020 coal-baseline import. The EB pathway creates a peak incremental demand of 133 MW at the Mararoa-Waimea GXP (EDN0331); BB creates 95 MW. Both signals are validated against EECA Southland RETA headroom estimates (Energy Efficiency and Conservation Authority 2022).

Regional screening (Layer 4): For each future ω, the incremental electricity signal is compared against available GXP headroom scaled by the headroom multiplier Uheadω. Where the signal exceeds headroom, the minimum sufficient upgrade is selected from four options (N+21, N+32, N+97, N+150 MW) with declared capital costs.

DMDU evaluation (Layer 6): System cost for each pathway under each future combines site operating cost and annualised upgrade capital cost (10-year amortisation at 7% real discount rate). Regret, satisficing rate, and win rate are computed across the 100-future ensemble.

### 5.3 Ensemble Design

The 100-future paired ensemble uses Latin Hypercube Sampling across six uncertain multipliers, treated as statistically independent draws. Multiplier ranges are calibrated against TIMES-NZ 3.0 Steady/Shift scenario bounds and EECA Southland RETA capacity estimates. Futures are paired so that both EB and BB are evaluated under identical multiplier draws, enabling unambiguous pathway comparison.

| Multiplier | P10 | Median | P90 |
| --- | --- | --- | --- |
| Electricity price | 0.785 | 0.987 | 1.184 |
| Biomass cost | 1.133 | 1.486 | 2.038 |
| ETS carbon price | 0.950 | 1.275 | 1.654 |
| GXP headroom capacity | 0.817 | 0.890 | 0.950 |
| Upgrade capital cost | 0.920 | 1.071 | 1.285 |
| Value of lost load | 10,000 | 15,000 | 20,000 NZD/MWh |

The independence assumption across multipliers is a simplifying approximation: in practice, electricity price and GXP headroom may be positively correlated through shared regional demand growth drivers. Relaxing this assumption is a sensitivity analysis that the progressive refinement framework would prioritise if the headroom multiplier continues to dominate regret sensitivity in subsequent ensemble designs.

### 5.4 Results

From within the facility boundary, the comparison appears as a cost-emissions trade-off with no infrastructure complexity. The BB pathway carries a private cost advantage of NZD 14.2M/year (NZD 76.8M versus NZD 91.0M annual site cost), while EB eliminates direct CO₂ emissions entirely. Both pathways require grid connection; neither triggers visible infrastructure costs within the site boundary. Conventional site- boundary analysis frames the decision as: choose BB for cost advantage, choose EB for zero direct emissions.

When the decision boundary includes regional grid infrastructure, the picture changes structurally. Under the EB pathway, 100 of 100 futures require a grid upgrade — 51 requiring N+97 MW (NZD 28.5M capex) and 49 requiring N+150 MW (NZD 52.0M capex, the major subtransmission reinforcement class). The BB pathway requires N+97 MW in all 100 futures but never triggers the N+150 MW class. The difference is the 38 MW gap in peak incremental demand between pathways (133 MW for EB, 95 MW for BB), which is sufficient to cross the N+150 MW upgrade threshold in 49% of futures under EB. The GXP headroom multiplier is the primary driver of this split: in futures where headroom is tighter, EB more frequently triggers the costlier upgrade class.

System-cost robustness results (Table 1) show BB preferred in 93 of 100 futures. EB’s satisficing rate is 35% versus BB’s 88%. EB’s maximum regret is NZD 18.9M/year; BB’s maximum regret is NZD 4.5M/year. EB carries 49% exposure to the N+150 MW upgrade class; BB carries zero.

Table 1: Robustness comparison — EB and BB pathways, 100-future paired ensemble, Edendale–Southland PoC

| Metric | EB | BB |
| --- | --- | --- |
| Annual site cost (NZD M/yr) | 91.0 | 76.8 |
| Direct CO₂ (tCO₂/yr) | 0 | 1,610 |
| Satisficing rate | 35% | 88% |
| Mean system regret (NZD M/yr) | 8.8 | 0.1 |
| Maximum regret (NZD M/yr) | 18.9 | 4.5 |
| N+150 MW upgrade exposure | 49% | 0% |
| System-cost win rate | 7% | 93% |

The private cost advantage of BB (NZD 14.2M/year lower site cost) is reinforced, not reversed, by the system- level perspective. The EB pathway’s infrastructure- conditional cost penalty — absent from site-boundary analysis — materialises when the decision boundary is drawn to include the regional grid.

## 6. Discussion

### 6.1 What the Boundary Placement Changes

The EB/BB comparison produces different recommendations depending on where the analytical boundary is drawn. This is the primary finding, and it has a specific mechanism: the 38 MW difference in peak incremental demand between pathways is sufficient to shift the grid upgrade class in 49% of futures. This upgrade class difference is the infrastructure-conditional consequence that site-boundary analysis cannot represent.

It is worth stating precisely what this result does and does not show. It does not show that EB is the wrong choice — in 7 of 100 futures, EB produces a lower system cost, and it eliminates direct emissions entirely. It shows that the comparison between pathways is materially different when infrastructure consequences are within the analytical scope, and that the difference favours BB in 93 of 100 futures under the declared ensemble design. The extent to which this result is robust to ensemble design choices — particularly the independence assumption between multipliers and the headroom multiplier range — should be tested in any operational application.

The result also illustrates the specific inadequacy of national-scale scenario analysis for this class of decision. TIMES-NZ 3.0’s Steady/Shift scenarios provide valuable macroeconomic framing, but neither scenario can resolve the GXP-level hosting capacity question that determines the upgrade class in any specific future. The 8,760-timestep hourly analysis at the GXP level is not a refinement of national modelling — it operates at a different analytical scale that the national model was not designed to replicate.

### 6.2 Limitations

Three limitations of the current PoC are declared in the analytical scope specification.

One-pass coupling. Site dispatch and regional screening run in sequence without iterative feedback. Upgrade costs do not influence technology dispatch decisions. Iterative multi-pass coupling would capture these feedback effects at the cost of computational tractability.

Proportional dispatch. The CFA dispatch rule produces the correct qualitative comparison ranking for a dairy processing facility with well-characterised operational patterns. Scheduling-grade dispatch would be needed for fine-grained operational analysis or for facilities with more complex demand profiles.

Stylised regional screening. The regional module uses capacity-multiplier scaling from EECA headroom estimates rather than a full network power flow model. The progressive refinement diagnostics identify this as the highest-priority development step: the headroom multiplier drives more of the EB/BB regret sensitivity than any other uncertain driver.

### 6.3 Generalisability

The framework’s three architectural principles — decision-first boundary specification, thin-waist artefact governance, ensemble evaluation under deep uncertainty — are domain-general. The Southland case demonstrates their application to industrial process heat decarbonisation. The same principles apply wherever long-horizon commitments create consequences that cross convenient physical system boundaries: water infrastructure planning, regional transport investment, corporate sustainability strategy, national energy transition pathways.

The structural condition that makes the DCM approach relevant is not specific to energy: it is present in any decision whose consequences propagate across scales and whose time horizon exceeds the predictive validity of probability-based forecasting.

## 7. Conclusions

Three conclusions follow from this analysis.

Analytical boundary placement is a consequential modelling choice. Drawing the boundary around the facility produces one recommendation; drawing it around the decision produces a materially different comparison in 93 of 100 plausible futures. The difference is not model error — it is the structural consequence of what the model was designed to see.

Modular architecture makes multi-scale decision analysis tractable. Each layer can be developed, validated, and replaced independently. Progressive refinement governed by regret diagnostics ensures analytical investment is directed where it most changes the decision-relevant comparison.

Regret, robustness, and satisficing are appropriate evaluative standards under deep uncertainty. When probability distributions over futures cannot be defended, expected value optimisation is not available as an evaluative standard. The DCM framework operationalises the alternative standards in a reproducible, auditable analytical chain.

The framework is open-access at https://ahmad-mahmoudi-coder.github.io/DCM/. Proof-of-concept data and pipeline documentation are available in the associated GitHub repository. Domain extensions to other infrastructure planning contexts are invited through the community contribution architecture documented at the platform.

## Acknowledgements

The author thanks supervisors Michael Protheroe and Michael Gschwendtner at Auckland University of Technology for guidance throughout the PhD research programme from which this work derives. The proof-of-concept analysis uses data from the Energy Efficiency and Conservation Authority (EECA) Regional Energy Transition Accelerator programme. The author is solely responsible for the analytical framework, its implementation, and the interpretations presented.

## References

Energy Efficiency and Conservation Authority. 2022. Regional Energy Transition Accelerator (RETA) Southland – Phase One Report.

Energy Efficiency and Conservation Authority. 2024a. Regional Heat Demand Database. https://www.eeca.govt.nz/insights/data-tools/regional-heat-demand-database/.

Energy Efficiency and Conservation Authority. 2024b. TIMES-NZ 3.0: New Zealand Energy Scenarios. https://www.eeca.govt.nz/insights/data-tools/new-zealand-energy-scenarios-times-nz/.

Haasnoot, Marjolijn, Jan H. Kwakkel, Warren E. Walker, and Jasper ter Maat. 2013. “Dynamic Adaptive Policy Pathways: A Method for Crafting Robust Decisions for a Deeply Uncertain World.” Global Environmental Change 23 (2): 485–98. https://doi.org/10.1016/j.gloenvcha.2012.12.006.

Herbst, Antje, Felipe Toro, Felix Reitze, and Eberhard Jochem. 2012. Introduction to Energy Systems Modelling.

Knight, Frank H. 1921. Risk, Uncertainty and Profit. Houghton Mifflin.

Kwakkel, Jan H., Marjolijn Haasnoot, and Warren E. Walker. 2016. “Comparing Robust Decision-Making and Dynamic Adaptive Policy Pathways for Model-Based Decision Support Under Deep Uncertainty.” Environmental Modelling and Software 86: 168–83. https://doi.org/10.1016/j.envsoft.2016.09.017.

Lahijani, Ahmad Mahmoudi, Mike D. Protheroe, and Michael Gschwendtner. 2025. “A Modelling Tool Selection for Decarbonising Industrial Process Heat Systems.” Renewable and Sustainable Energy Reviews 210: 115149. https://doi.org/10.1016/j.rser.2024.115149.

Lempert, Robert J., Steven W. Popper, and Steven C. Bankes. 2003. Shaping the Next One Hundred Years: New Methods for Quantitative, Long-Term Policy Analysis. RAND Corporation.

Marchau, Vincent A. W. J., Warren E. Walker, Pieter J. T. M. Bloemen, and Steven W. Popper. 2019. Decision Making Under Deep Uncertainty: From Theory to Practice. Springer. https://doi.org/10.1007/978-3-030-05252-2.

Pfenninger, Stefan, Joseph DeCarolis, Lion Hirth, Sylvain Quoilin, and Iain Staffell. 2018. “The Importance of Open Data and Software: Is Energy Research Lagging Behind?” Energy Policy 101: 211–15. https://doi.org/10.1016/j.enpol.2016.11.046.

Powell, Warren B. 2022. Sequential Decision Analytics and Modeling. Now Publishers. https://castlelab.princeton.edu/SDAM.

Simon, Herbert A. 1955. “A Behavioral Model of Rational Choice.” Quarterly Journal of Economics 69 (1): 99–118. https://doi.org/10.2307/1884852.